CHAPTER 1

# ABOUT THE ORGANIZATION

## Indian Space Research Organization (ISRO)

The Indian Space Research Organization (ISRO) is the national space agency of India, headquartered in Bengaluru. One of the world's leading space organizations, ISRO's primary objective is to harness space technology for national development while pursuing space science research and planetary exploration. Over the decades, ISRO has consistently demonstrated its prowess in developing indigenous launch vehicles, satellites for communication, remote sensing, and navigation, making space accessible and beneficial to the common citizen.

History and Background

The Indian Space Research Organisation (ISRO) is India's national space agency, headquartered in Bengaluru. It was officially formed on August 15, 1969, under the leadership of Dr. Vikram Sarabhai, known as the "Father of the Indian Space Programme".

Origin and Foundation

- INCOSPAR (1962): Modern space research in India began with the setting up of the Indian National Committee for Space Research (INCOSPAR) by the Government of India in 1962. It was established under the Department of Atomic Energy (DAE) at the suggestion of Dr. Sarabhai.

- Thumba Rocket Launching (1963): INCOSPAR established the Thumba Equatorial Rocket Launching Station (TERLS) in Kerala. The first sounding rocket, a Nike-Apache supplied by NASA, was launched from here on November 21, 1963.

- ISRO Formation (1969): ISRO superseded INCOSPAR in 1969 to better coordinate and expand the country's space activities.

- Department of Space (1972): In 1972, the Government of India established the Space Commission and the Department of Space (DoS), bringing ISRO under its administrative control.

Key Historical Milestones

ISRO's history is often divided into phases:

- Experimental Phase (1970s–1980s):

  - Aryabhata (1975): India's first satellite, Aryabhata, was launched using a Soviet rocket in April 1975.

  - SLV-3 (1980): India's first indigenous satellite launch vehicle, the SLV-3, successfully placed the Rohini satellite into orbit in 1980, making India the seventh nation to achieve orbital launch capability.

- Operational Phase (1990s–Present):

  - The Workhorse (PSLV): The Polar Satellite Launch Vehicle (PSLV) became operational in the 1990s, becoming ISRO's most reliable launcher for Earth observation satellites.

  - GSLV and Cryogenics: ISRO developed the Geosynchronous Satellite Launch Vehicle (GSLV) to launch heavier communication satellites, eventually mastering indigenous cryogenic engine technology in 2014. [1, 2, 8, 9, 10, 11]

Recent Major Achievements

- Chandrayaan Series: Chandrayaan-1 (2008) discovered evidence of water on the Moon, while Chandrayaan-3 (2023) made India the first nation to soft-land near the lunar south pole.

- Mars Orbiter Mission (Mangalyaan): Launched in 2013, it made India the first country to reach Mars orbit on its maiden attempt.

- Solar Mission (Aditya-L1): Launched in September 2023, it is India's first space-based mission to study the Sun.

Figure 1.1 : ISRO: The India’s Space Agency

1.2  ISRO Telemetry Tracking and Command Network

The ISRO Telemetry, Tracking and Command Network (ISTRAC) is the operational backbone for all of   India's space missions, from low-Earth orbit satellites to interplanetary probes. As of early 2026, ISTRAC is undergoing a significant modernization phase to support the upcoming Gaganyaan human spaceflight and future planetary explorations.

Strategic Infrastructure & Network

Headquartered in Bengaluru, ISTRAC operates a globally distributed network of ground stations that provide 24/7 visibility for satellites:

- Domestic Stations: Bengaluru, Lucknow, Sriharikota, Thiruvananthapuram, and Port Blair.

- International Stations: Mauritius, Brunei, Biak (Indonesia), and ground support links in Norway (Svalbard) and Antarctica (Troll).

- Indian Deep Space Network (IDSN): Located at Byalalu (near Bengaluru), this facility features massive 32-metre and 18-metre antennas specifically designed to communicate with deep-space missions like Chandrayaan-3, Aditya-L1, and the future Venus Orbiter.

Core Mission Responsibilities

- Mission Control (MOX): The Mission Operations Complex (MOX) in Bengaluru is the nerve centre where health analysis, orbit determination, and satellite maneuvers are executed.

- Navigation (NavIC): ISTRAC manages the ground segment for the Navigation with Indian Constellation (NavIC), maintaining the high-precision atomic clocks and 21 ranging stations required for independent regional positioning.

- Space Situational Awareness (IS4OM): The ISRO System for Safe and Sustainable Space Operations Management, inaugurated at ISTRAC, monitors space debris to safeguard Indian assets from potential collisions.

- Search and Rescue (SAR): As part of an international programme (COSPAS-SARSAT), ISTRAC provides satellite-aided search and rescue services by processing distress signals from across the region.

2025–2026 Modernization & Future Goals

- Gaganyaan Support: ISTRAC has recently integrated its mission control with the European Space Agency's (ESA) ground network to ensure uninterrupted communication with Indian astronauts during the Gaganyaan orbital module's flight.

- Technology Upgrades: Existing 10-metre terminals are being replaced with 11-metre multiband terminals capable of handling X-band and Ka-band frequencies to manage higher data volumes.

- Space Docking Success: In January 2025, ISTRAC successfully managed the SpaDeX mission, demonstrating India's first-ever in-space docking, a critical step for building the planned Bharatiya Antariksh Station (Indian Space Station) by 2035.

- AI Integration: The agency is implementing AI-driven algorithms for autonomous mission operations and automated tracking of large satellite constellations.

Figure 1.2 : 32m diameter antenna- for MOM

  - ISTRAC’s Role in NavIC

The ISRO Telemetry, Tracking and Command Network (ISTRAC) serves as the central intelligence and operational core for NavIC (Navigation with Indian Constellation). Beyond simple monitoring, ISTRAC manages the complex interplay of high-precision timing, orbital physics, and signal integrity necessary for independent regional navigation.

1. Navigation Control Centre (INC) & Data Generation

The INC at ISTRAC is the "brain" of the ground segment. Its primary role is to ensure the Navigation Message is accurate and synchronized.

- Ephemeris & Clock Updates: ISTRAC periodically calculates the precise orbital position (ephemeris) and the infinitesimal drift in the onboard atomic clocks of each satellite. These corrections are vital because even a nanosecond of timing error can lead to significant positioning inaccuracies on the ground.

- Data Upload: These navigation parameters, along with the almanac (coarse orbital data for the entire constellation), are periodically uplinked to the satellites to be broadcast to users.

2. IRNSS Network Timing (IRNWT) Facility

ISTRAC operates the IRNWT facility, which generates a stable reference time called IRNWT Time.

- Ensemble of Clocks: This time is maintained by an ensemble of high-performance Active Hydrogen Masers and Cesium atomic clocks.

- System Synchronization: This ground-based reference is used to synchronize the entire constellation, ensuring that "satellite time" and "ground time" remain unified, which is the fundamental requirement for trilateration in navigation.

3. Range and Integrity Monitoring Stations (RIMS)

RIMS are geographically distributed stations that act as "trusted users". They do not just receive the signal; they scrutinize it.

- Real-time Monitoring: RIMS continuously track NavIC satellites to provide one-way ranging data used for orbit determination.

- Integrity Checks: They monitor for signal anomalies, such as unexpected shifts in frequency or power. If a satellite's signal falls outside safety limits, ISTRAC can trigger an "unhealthy" status bit in the navigation message, warning users within seconds.

4. Signal Structure & Ranging Accuracy

The accuracy of the distance (range) between a satellite and a user depends on the autocorrelation properties of the spreading codes (PRN codes).

- Spreading Codes: NavIC uses sophisticated codes, such as the Interleaved Z4 (IZ4) family for the L1 band, designed to have superior correlation performance.

- Correlation & Ranging: ISTRAC’s RIMS use these properties to precisely identify the "time of arrival" of a signal. Any degradation in the signal—caused by hardware aging or atmospheric interference—blunts the autocorrelation peak, making it harder to pinpoint the exact time of arrival and thus increasing the error in the calculated position.

- Ionospheric Grid Corrections: Unique to NavIC, ISTRAC calculates and broadcasts ionospheric grid corrections for 80 specific points over India. This allows single-frequency users to achieve much higher accuracy by correcting for atmospheric delays that typically degrade GNSS signals

CHAPTER 2

OBJECTIVES OF THE INTERNSHIP

2.1    Primary Objectives

1. CDMA Architecture and PRN Codes

In a Code Division Multiple Access (CDMA) system, all satellites transmit on the same frequency simultaneously. The "magic" that prevents interference is the Pseudo-Random Noise (PRN) code. ISTRAC ensures that each NavIC satellite is assigned a unique, orthogonal PRN sequence. These sequences act as digital signatures, allowing a receiver to pick out a specific satellite's signal from below the noise floor using a correlation process.

2. Abstract Algebra & Galois Fields ((GF(2n)))

The generation of these codes isn't random; it is rooted in Abstract Algebra:

- Groups & Rings: These provide the rules for binary arithmetic (XOR operations).

- Galois Fields ((GF(2)): NavIC sequences are generated in a finite field of order 2. To create a code of length (N = 2(n - 1), we operate within (GF(2n)).

- Polynomials: The "recipe" for a sequence is a Primitive Polynomial. In (GF(2n)), a primitive polynomial of degree (n) is essential because it ensures the sequence does not repeat until it has cycled through every possible state (except all zeros).

3. LFSRs and m-sequences

A Linear Feedback Shift Register (LFSR) is the hardware/software mechanism that implements these polynomials.

- Maximum Length Sequences (m-sequences): These are the longest possible sequences an \(n\)-bit LFSR can produce (2n-1).

- Properties: They possess the Balance Property (nearly equal 0s and 1s) and the Shift-and-Add Property, making them the building blocks for more complex codes.

4. NavIC Gold Codes (SPS)

While m-sequences are great, there are very few of them for a given length. Gold Codes solve this by XORing two specifically chosen "preferred" m-sequences.

- The Benefit: Gold codes provide a much larger set of sequences with uniform and predictable cross-correlation properties, which is vital for NavIC to prevent one satellite's signal from being mistaken for another (the "near-far" problem).

2.2  Core Project Objectives: Implementation & Optimization

1. Python Implementation

Using Python, you can model the hardware logic of an LFSR. Libraries like NumPy are used to handle bitwise operations at scale, allowing you to simulate the generation of the 1023-bit codes used in NavIC’s L5 and S-band signals.

2. Statistical Analysis of Binary Sequences

To be "GPS-like," a sequence must pass three randomness tests:

- Balance Property: The number of 1s and 0s should differ by at most one.

- Run-length Property: Half the runs (consecutive identical bits) should be of length 1, a quarter of length 2, and so on.

- Correlation: This is the most critical. You must calculate the Periodic Autocorrelation Function (PACF) to ensure the signal is easily "lockable."

3. The "Sidelobe" Problem

In an ideal world, the autocorrelation of a sequence would be (N) at zero shift and (0) everywhere else. In reality, sequences have sidelobes (peaks at non-zero shifts).

- The Risk: High out-of-phase sidelobes can cause a receiver to "false lock" onto a reflected signal (multipath) or a shifted version of the code, leading to massive errors in the calculated distance (ranging).

4. Optimization: Minimizing PSL and ISL

Standard Gold codes are mathematically sound but not always optimal for every environment. Your objective involves:

- Peak Sidelobe Level (PSL): The highest "fake" peak in the correlation. Minimizing this reduces false locks.

- Integrated Sidelobe Level (ISL): The total energy in all sidelobes. Minimizing this improves the Signal-to-Noise Ratio (SNR).

- Algorithms: You will implement optimization techniques—such as Genetic Algorithms, Simulated Annealing, or Cyclic Algorithms (like the WeCAN algorithm)—to search for "Sub-Gold" or "Custom" sequences that outperform standard NavIC codes in specific interference scenarios.

CHAPTER 3

LEARNING EXPERIENCES

3.1  Overview of NavIC (IRNSS)

NavIC (Navigation with Indian Constellation) is India's autonomous regional satellite navigation system. Developed by ISRO, it provides accurate real-time positioning and timing services over India and a region extending approximately 1,500 km around its boundaries.

- Space Segment: Consists of a constellation of 7-plus satellites in Geostationary (GEO) and Geosynchronous (GSO) orbits.

- Ground Segment: Managed by ISTRAC, responsible for maintenance, satellite control, and navigation parameter generation.

- Service Types:

  - SPS (Standard Positioning Service): For civilian use.

  - RS (Restricted Service): Encrypted service for authorised/military users.

Core Technical Competencies Gained

A. Digital Communication & CDMA Architecture

I gained a deep understanding of Code Division Multiple Access (CDMA), the technology that allows all NavIC satellites to transmit on the same frequency (L5 and S-band).

- Learning: I mastered how unique PRN (Pseudo-Random Noise) codes are used to "spread" the signal, providing processing gain and allowing receivers to distinguish between different satellites through correlation.

B. Advanced Mathematics: Galois Fields & Abstract Algebra

NavIC's reliability is rooted in Finite Field theory. I explored the mathematical foundations required to build these signals:

- Galois Fields (\(GF(2^n)\)): Learned how to perform binary arithmetic (XOR/AND) within finite fields to generate deterministic sequences.

- Primitive Polynomials: Studied these "mathematical keys" to ensure that an LFSR produces a Maximum Length Sequence (m-sequence) without premature repetition.

C. Sequence Generation and LFSR Logic

I studied the hardware and software implementation of Linear Feedback Shift Registers (LFSRs).

- m-sequences: Understood their near-perfect autocorrelation but limited cross-correlation properties.

- Gold Codes: Deep-dived into the generation of Gold Codes (used in NavIC SPS) by XORing preferred pairs of m-sequences. I learned why NavIC uses a code length of 1023 chips (n=10) to balance acquisition speed and interference rejection.

D. Signal Integrity & Correlation Analysis

A major part of my experience was analyzing why some signals "lock" better than others.

- Autocorrelation: Learned that a sharp central peak is vital for accurate "Time of Flight" calculation.

- Sidelobe Management: Studied how high Peak Sidelobe Levels (PSL) can lead to "false locks," especially in multipath environments (like urban canyons), resulting in ranging errors.

Practical Implementation: Python for SatNav

I translated theoretical mathematics into functional software tools using Python:

- Modular Coding: Developed modules to simulate LFSRs and generate Gold Code families.

- Scientific Computing: Utilized NumPy and SciPy for FFT-based correlation, reducing the computational time required to analyze long binary sequences.

- Visualization: Created plots to visualize autocorrelation peaks and power spectral density, which are critical for validating signal quality.

Optimization & Research Focus

The pinnacle of the learning experience involved reframing signal design as a combinatorial optimization problem.

- Problem Solving: I investigated methods to go beyond standard algebraic Gold codes to find sequences with lower Integrated Sidelobe Levels (ISL).

- Algorithmic Thinking: Explored optimization heuristics (like Simulated Annealing or Genetic Algorithms) to minimize interference, directly contributing to the goal of enhancing NavIC’s ranging accuracy.

Professional & Strategic Insights

- Self-Reliance: Understood the strategic importance of NavIC for India's Atmanirbhar Bharat (Self-reliant India) initiative, reducing dependency on foreign GNSS (like GPS or GLONASS).

- System Reliability: Gained insight into how ISTRAC monitors signal integrity in real-time through RIMS (Range and Integrity Monitoring Stations) to ensure user safety.

Figure 3.1 :  Working of NavIC : Indian Navigation System

3.2  Mathematical Foundations: Groups, Rings, and Galois Fields

3.2.1 Groups

A Group (G, *) is a set of elements combined with a single operation (*), which could be addition or multiplication) that follows four strict rules:

- Closure: If you pick any two elements in the set and apply the operation, the result stays in the set.

- Associativity: (a * b) * c = a * (b * c).

- Identity Element: There is a "neutral" element (like (0) in addition or (1) in multiplication) that doesn't change other elements.

- Inverse Element: Every element has a "partner" that brings it back to the identity (like (5) and (-5) in addition).

In NavIC/SatNav: We use Abelian Groups (where (a*b = b*a)). The simplest group used is the set ({0, 1}) under XOR addition.

3.2.2  Rings

A Ring (R, +, *) is a set that has two operations, usually called addition and multiplication.

- Under Addition, it must be an Abelian Group (has identity (0) and inverses).

- Under Multiplication, it only needs to be associative and distributive over addition (a * (b + c)).

- Crucial Difference: In a Ring, you might not be able to "divide" (not every element has a multiplicative inverse).

In NavIC/SatNav: We work with Polynomial Rings. When we design an LFSR, we are essentially performing math in a ring where the elements are polynomials.

3.2.3  Galois Fields

A Field is a Ring where division is possible (except by zero). Every non-zero element must have a multiplicative inverse. A Galois Field (GF) is simply a field with a finite number of elements.

GF(2): The Binary Field

This is the heart of all digital logic and ISRO's PRN codes.

- Elements: {0, 1}

- Addition (XOR): (1+1=0, 1+0=1, 0+0=0).

- Multiplication (AND): (1*1=1, 1*0=0, 0*0=0).

GF(2n) : The Extension Field

To create the 1023-bit codes for NavIC, we use GF(210).

- Elements: These aren't just single bits; they are represented as polynomials of degree (n-1).

- Primitive Polynomials: To stay within the "Finite" field, we use a special polynomial (the Primitive Polynomial) as a modulus. Much like (13 mod 12 = 1) on a clock, we divide our results by this polynomial to keep them within the field.

3.3  PRN Codes

Pseudo-Random Noise (PRN) codes are deterministic binary sequences that appear statistically random. In satellite navigation systems like NavIC and GPS, they are the fundamental mechanism for identifying satellites and calculating the distance (range) between the satellite and the receiver.

Functional Roles of PRN Codes

- Satellite Identification: Each satellite in a constellation transmits a unique PRN code. Using Code Division Multiple Access (CDMA), multiple satellites can transmit on the same frequency simultaneously without interfering, as the receiver uses these unique codes to distinguish between them.

- Distance (Range) Calculation: The receiver generates a local replica of a satellite's PRN code and shifts it in time until it matches the incoming signal. This time shift represents the signal's travel time from the satellite to the receiver, which is multiplied by the speed of light to determine the pseudo range.

- Spread Spectrum & Jamming Resistance: PRN codes "spread" the navigation data over a wide bandwidth. This makes the signal highly resistant to narrowband interference and jamming, as the "despreading" process at the receiver essentially filters out non-matching noise.

Signal Structure and Generation

PRN codes are typically generated using Linear Feedback Shift Registers (LFSRs).

- Coarse Acquisition (C/A) Codes: Standard civilian signals (like those on GPS L1 or NavIC L5/S-bands) use Gold codes. For most civilian services, these are 1,023 chips long and repeat every 1 millisecond at a rate of 1.023 MHz.

- Precise (P) Codes: Used for military applications, these codes are much longer (often trillions of bits) and can take a full week to repeat, providing higher accuracy and encryption (Y-code).

- NavIC Specifics: NavIC utilizes PRN sequences for its SPS (Standard Positioning Service) on the L5 (1176.45 MHz) and S-band (2492.028 MHz). While it primarily uses Gold codes for SPS, it is evolving to include more advanced sequences like IZ4 codes for the L1 band to improve performance.

Key Mathematical Properties

The efficiency of a PRN code is determined by its correlation properties:

- Autocorrelation: A code should have high correlation with itself only at zero phase difference (perfect alignment). This results in a sharp "spike" or thumbtack peak, which allows the receiver to pinpoint the exact time of arrival.

- Cross-correlation: Different codes in the same set should have extremely low correlation with each other. This ensures that a receiver looking for Satellite A does not accidentally lock onto the signal from Satellite B.

- Balance & Run-length: Ideally, the number of 1s and 0s in a code should be nearly equal (balance property) to prevent DC bias in the signal.

Code Generation

A. The Core Engine: Linear Feedback Shift Register (LFSR)

The generation of a PRN code is performed by a digital circuit called an LFSR. It consists of two main parts:

- Shift Register: A series of memory cells (flip-flops) that hold bits (0 or 1).

- Feedback Logic: A network that takes bits from specific positions (called taps) and combines them using XOR gates (modulo-2 addition) to create a new bit that is fed back into the start of the register.

B. The Step-by-Step Generation Process

Imagine an LFSR with 10 stages (standard for many navigation signals):

- Initialisation (Seed): The register is loaded with an initial set of bits (e.g., all 1s). It cannot be all 0s, or the register would get stuck.

- Tapping: Specific stages are "tapped" based on a mathematical formula.

- Feedback Calculation: The bits at the tap positions are XORed together.

  - Rule: If the number of 1s at the taps is odd, the feedback is 1; if even, it is 0.

- Shifting:

  - The bit in the very last stage is shifted out—this is your PRN output bit.

  - Every other bit moves one position to the right.

  - The new feedback bit is moved into the first stage.

- Iteration: This happens millions of times per second (e.g., at 1.023 MHz).

Figure 3.3: PRN Code Generator

C. The Mathematical "Key": Primitive Polynomials

The choice of which stages to tap is not random. It is determined by Primitive Polynomials from Galois Field theory.

- If you choose the taps correctly (using a primitive polynomial), an \(n\)-bit register will cycle through every possible combination of bits (except all 0s).

- The resulting sequence is called an m-sequence.

- Length formula: The code will have a length of (N = 2n -1) bits before it repeats. For a 10-bit register, this is (210- 1 = 1023) bits.

D. Why this "Pseudo-Random" Logic is used

Even though the process is completely deterministic (if you know the seed and the taps, you know the code), the output has properties of random noise:

- Balance: There is almost an equal number of 1s and 0s (the number of 1s is always one more than the number of 0s).

- Run Property: The patterns of consecutive 1s and 0s follow a predictable distribution similar to a coin toss.

- Correlation: If you shift the sequence by even one bit, it no longer looks like itself. This "sharp" autocorrelation is what allows ISRO/ISTRAC to measure the distance to a satellite with meter-level precision.

E. Summary of the Logic

To generate a PRN code, you simply need:

- A Register length (\(n\)): Defines the complexity.

- A Primitive Polynomial: Defines the "tap" positions.

- A Clock: Defines how fast the bits are produced.

3.4  LFSR

A. Introduction to LFSR Technology

A Linear Feedback Shift Register (LFSR) is a specialized hardware or software architecture used to generate sequences of binary bits that appear statistically random but are entirely deterministic. In the context of the Indian Space Research Organisation (ISRO) and the NavIC constellation, the LFSR is the fundamental "signal engine." It is responsible for creating Pseudo-Random Noise (PRN) codes, which allow for satellite identification, spread spectrum communication, and high-precision ranging.

The "Linear" in LFSR refers to the fact that the input bit is a linear function of its previous state—specifically, it is generated using modulo-2 addition (XOR logic). The "Shift Register" refers to the movement of bits through a series of memory cells triggered by a high-frequency clock.

B. The Physical Architecture and Components

To understand an LFSR, one must analyse its three primary physical constituents:

Stages and Memory Cells

The register consists of \(n\) stages, where each stage is typically a D-type flip-flop capable of storing a single binary bit (0 or 1). The number of stages, (n), defines the "order" of the LFSR. For NavIC’s Standard Positioning Service (SPS), (n) is 10.

- State Space: An \(n\)-stage register has (2n possible states. However, because the feedback is linear, the "all-zero" state is a terminal state (null state). Therefore, the functional state space is (2n - 1).

- Memory Efficiency: Unlike a memory-based system that would need to store a 1024-bit string, an LFSR "calculates" the string on the fly using only 10 bits of physical memory.

Taps and Feedback Paths

Taps are specific stages in the register whose values are "tapped off" to be used in the feedback calculation. The location of these taps is not arbitrary; they are defined by a Characteristic Polynomial. If a tap is "active," its value is sent to the XOR network; if "inactive," it is ignored.

XOR Feedback Logic

The Exclusive OR (XOR) gate is the heart of the LFSR. It implements modulo-2 addition. The logic is simple: if the number of 1s at the input taps is odd, the output is 1; if even, the output is 0. This ensures that the feedback bit is always a linear combination of the current register state.

C. Operational Mechanics: The "Shift-and-Feedback" Cycle

The LFSR operates in a discrete-time environment governed by a master clock. In the NavIC system, this clock runs at a "chipping rate" of 1.023 MHz. During each clock cycle, three events occur simultaneously:

- Bit Output: The bit currently residing in the \(n^{th}\) (last) stage is shifted out of the register. This bit becomes one "chip" of the PRN sequence transmitted to Earth.

- State Shifting: Every bit within the register moves one position to the right ((S_1 to S_2, S_2 to S_3), etc.).

- Feedback Insertion: The XOR gate calculates the new bit based on the values of the tapped stages before they shifted. This new bit is then inserted into the first stage ((S_{1})), filling the vacancy and preparing the register for the next cycle.

D. Mathematical Foundation: Galois Field Theory

The behavior of an LFSR is deeply rooted in Abstract Algebra, specifically Galois Fields of order 2 ((GF(2))).

Primitive Polynomials

For an LFSR to be useful in navigation, the polynomial must be Primitive. A primitive polynomial is irreducible (cannot be factored) and possesses the mathematical property required to generate all elements of an extension field.

- Maximum Length (m-sequences): A primitive polynomial ensures the LFSR visits every possible non-zero state before repeating. The resulting output is known as an m-sequence.

- Cycle Length: For NavIC, \(n=10\), so the m-sequence length is (210- 1 = 1023) bits.

The All-Zero Constraint

Mathematically, if an LFSR ever enters the state where every cell is 0, the XOR sum of those cells will always be 0. Consequently, the register will be "stuck" in a perpetual loop of zeros. To prevent this, ISRO engineers ensure that the Seed (initial state) is always a non-zero value (typically all 1s).

E. Hardware Architectures: Fibonacci vs. Galois

In high-level design, two distinct architectures are used to implement the LFSR logic:

Fibonacci Configuration (Many-to-One)

In this classic design, the feedback is external. Bits are taken from various taps, XORed in a chain, and the final result is fed back into the first stage.

- Pros: Intuitive and matches the mathematical polynomial directly.

- Cons: "Propagation Delay." As the number of taps increases, the signal must pass through multiple XOR gates in series. This limits the maximum clock speed of the hardware.

Galois Configuration (One-to-Many)

In the Galois architecture, the output of the last stage is fed back to all tap positions simultaneously. XOR gates are placed in-between the stages.

- Pros: Parallel processing. There is only one gate delay between any two flip-flops, regardless of the number of taps.

- ISRO Utility: This is the industrial standard for ISTRAC ground hardware because it allows for ultra-high-speed signal generation with maximum stability.

F. Strategic Properties and Performance Metrics

Why does NavIC rely on the LFSR? The answer lies in the statistical properties of the output sequence.

Balance Property

In a full cycle of an m-sequence, the number of 1s is always exactly one more than the number of 0s. This near-perfect balance ensures that the signal has no DC component, which is critical for efficient radio frequency (RF) transmission.

Run-Length Property

The distribution of "runs" (consecutive 1s or 0s) in an LFSR sequence mimics random noise. This "whiteness" of the spectrum ensures that the energy is spread evenly across the bandwidth, making the signal difficult to jam or intercept.

Periodic Autocorrelation (The Ranging Secret)

This is the most vital property for ISTRAC.

- The Peak: When the received satellite signal is perfectly aligned with the receiver's local LFSR replica, the correlation value is 1023.

- The Floor: If the signal is shifted by even a single bit (one chip), the correlation value drops to -1.

- Significance: This massive 1023-to-1 ratio allows the NavIC receiver to pull a signal out of background space noise that is thousands of times stronger than the signal itself.

G. Real-World Implementation in NavIC (SPS)

In the Standard Positioning Service (SPS) provided by NavIC:

- Chipping Rate: The LFSR runs at 1.023 million chips per second.

- Spatial Precision: Each "chip" corresponds to approximately 293 meters of distance. By using the LFSR's sharp correlation peak, receivers can resolve the timing to within a fraction of a chip, enabling positioning accuracy of less than 5 meters.

- Satellite Identity: Each satellite is assigned a unique phase or polynomial, allowing the receiver to differentiate between Satellites 1 through 7 even though they all talk at once.

The Linear Feedback Shift Register is more than just a bit-shifter; it is a masterclass in efficient engineering. By combining the rigid laws of Galois Field theory with simple digital flip-flops, ISRO is able to create a global positioning infrastructure that is fast, low-power, and mathematically unshakeable. For an intern at ISTRAC, mastering the LFSR is the gateway to understanding the entire chain of satellite communication and deep-space navigation.

3.5  Properties of the sequences used in Satellite Communication

In satellite communication, particularly for systems like NavIC and GPS, we don't just use any random string of 1s and 0s. We use Pseudo-Random Noise (PRN) sequences (typically m-sequences or Gold codes) because they possess specific mathematical properties that allow a receiver to pick up a tiny signal from thousands of kilometres away.

1. The Balance Property

In any full cycle of a maximal length sequence (m-sequence), the number of ones is always exactly one more than the number of zeros.

- The Math: For a sequence of length (N = 2n - 1), there are ((N+1)/2) ones and ((N-1)/2) zeros.

- Why it matters: This ensures the signal is "DC-balanced." If there were too many 1s or 0s, the signal would have a constant voltage offset, which wastes power and makes it much harder for satellite transmitters to send the signal efficiently over long distances.

2. The Run-Length Property

A "run" is a string of identical consecutive bits (e.g., 111 or 00). In a PRN sequence, the distribution of these runs is strictly defined:

- Half the runs have a length of 1.

- One-fourth of the runs have a length of 2.

- One-eighth of the runs have a length of 3, and so on.

- Why it matters: This distribution mimics "White Noise." It ensures that the signal energy is spread evenly across the entire frequency spectrum. This is known as Spread Spectrum technology, which makes the NavIC signal highly resistant to intentional jamming or natural interference.

3. The Autocorrelation Property (The "Click" Effect)

This is the most critical property for Ranging (calculating distance). Autocorrelation measures how much a sequence matches a time-shifted version of itself.

- The Result: When the two sequences are perfectly aligned, the correlation is maximum ((N)). If they are shifted by even a single bit (one chip), the correlation drops to a very low constant value (-1).

- Why it matters: This creates a sharp "thumbtack" peak. When your receiver (or ISTRAC ground station) slides its internal code and finds this peak, it knows the exact nanosecond the signal arrived. This timing precision is what allows NavIC to tell you your location within a few metres.

4. The Cross-Correlation Property

In satellite navigation, all satellites transmit on the same frequency at the same time. Cross-correlation measures how much one satellite's code (e.g., NavIC Sat 1) matches another satellite's code (e.g., NavIC Sat 2).

- The Requirement: The cross-correlation must be extremely low (near zero).

- Why it matters: This allows for Satellite Identification. It prevents a receiver from "false-locking" onto the wrong satellite. Even though seven different signals are hitting your antenna, the low cross-correlation acts like a filter, allowing the receiver to hear only the "voice" of the specific satellite it is looking for.

5. Shift-and-Add Property

If you take an m-sequence and XOR it with a time-shifted version of itself, the result is simply another time-shifted version of the same sequence.

- Why it matters: This property is used mathematically to generate Gold Codes. By XORing two different m-sequences (a "preferred pair"), we can create a much larger family of codes that all maintain the low cross-correlation required for a full constellation like NavIC.

Example:

let’s use a 3-stage LFSR.

- Length n: 3

- Max Length N: (23 - 1 = 7) bits.

- Primitive Polynomial: (1 + X + X3) (This means we tap Stage 1 and Stage 3).

- Initial Seed: 1 1 1

The Step-by-Step Generation

In each clock cycle, we XOR the bits in Stage 1 and Stage 3 to get the Feedback, then Shift everything right.

| Clock Cycle | Stage 1 | Stage 2 | Stage 3 | Output (Bit out) | Feedback (S1 ⊕ S3) |
| --- | --- | --- | --- | --- | --- |
| Start | 1 | 1 | 1 | — | 1 ⊕ 1 = 0 |
| 1 | 0 | 1 | 1 | 1 | 0 ⊕ 1 = 1 |
| 2 | 1 | 0 | 1 | 1 | 1 ⊕ 1 = 0 |
| 3 | 0 | 1 | 0 | 1 | 0 ⊕ 0 = 0 |
| 4 | 0 | 0 | 1 | 0 | 0 ⊕ 1 = 1 |
| 5 | 1 | 0 | 0 | 1 | 1 ⊕ 0 = 1 |
| 6 | 1 | 1 | 0 | 0 | 1 ⊕ 0 = 1 |
| 7 | 1 | 1 | 1 | 0 | 1 ⊕ 1 = 0 |
| 8 | 0 | 1 | 1 | 1 | (Repeats Cycle 1) |

Table 3.5 :  LFSR step by step calculation

The resulting PRN Sequence is: 1 1 1 0 1 0 0

How this illustrates the Properties:

- Balance Property:

  - Count the 1s: 4

  - Count the 0s: 3

  - Result: There is exactly one more '1' than '0'. (Balanced).

- Run-Length Property:

  - Run of three 1s: 111 (length 3)

  - Run of two 0s: 00 (length 2)

  - Single bits: 1 and 0

  - Result: The distribution matches the mathematical rule for randomness.

- Autocorrelation :

  - If you compare 1110100 with itself, all 7 bits match.

  - If you shift it by one (0111010), only 3 bits will match.

  - The drop from 7 to 3 is what the receiver uses to find the exact "start" of the signal.

- Deterministic Nature:

  - Because we used the seed 111 and the taps 1, 3, we will always get 1110100.

3.6  Primitive Polynomials

1. Defining the Primitive Polynomial

A polynomial (P(x)) of degree (n) is considered primitive if it meets three strict criteria in Galois Field theory ((GF(2))):

- Irreducible: It cannot be factored into smaller polynomials (similar to a "prime number" in the world of equations).

- Degree (n): The highest power of (x) is (n) (for NavIC SPS, (n=10)).

- Maximum Period: It is a generator for the field (GF(2n)). This means its roots have the property that they generate every non-zero element in the field. Mathematically, it must divide (xk-1) for (k = 2n - 1\), and for no smaller integer (k).

2. Role in LFSR Generation

The exponents in the polynomial tell you exactly where to place the taps on your LFSR.

- Example: If the polynomial is (1 + x3 + x10), you place XOR taps at the 3rd and 10th stages of the register.

- The Outcome: Because the polynomial is primitive, the LFSR is guaranteed to produce an m-sequence (Maximal Length Sequence). It will cycle through (210 - 1 = 1023) states before it

ever repeats a single bit.

3. Why "Primitive" is Essential for SatNav

If you used a polynomial that was not primitive:

- Short Cycles: The LFSR might repeat every 20 or 50 bits instead of 1023. This would cause "aliasing," where the receiver thinks the satellite is at multiple distances simultaneously.

- Poor Randomness: The distribution of 1s and 0s would be skewed, meaning the signal wouldn't look like noise (Spread Spectrum). This would make the signal very easy to jam.

4. Connection to Galois Fields ((GF(2n)))

A primitive polynomial defines the arithmetic of the field. Every state of your 10-bit LFSR can be thought of as a polynomial of degree 9.

- Shifting the LFSR is mathematically equivalent to multiplying by \(x\) in the field.

- The feedback (XOR) is equivalent to taking the modulus by the primitive polynomial.

- This ensures the "sequence" stays within the finite field boundaries while visiting every possible point.

5. Application in NavIC and GPS

The choice of primitive polynomial is a design signature:

- GPS L1 C/A Code: Uses two polynomials: (1 + x3 + x10) and (1 + x2 + x3 + x6 + x8 + x9 + x10).

- NavIC: Also utilizes specific degree-10 primitive polynomials to generate its unique Gold code families.

3.7  Applications of Primitive Polynomials

Beyond satellite navigation, Primitive Polynomials are the mathematical engines behind almost every modern digital communication and security system. Because they can generate "random-looking" but perfectly predictable sequences, they are used wherever data needs to be scrambled, protected, or checked for errors.

Here are the key applications to include in your documentation:

1. Error Detection and Correction (CRC)

The most common use of primitive polynomials is in Cyclic Redundancy Checks (CRC).

- How it works: When data is sent over a network (like Ethernet or Wi-Fi), it is treated as one long polynomial. This data is divided by a primitive generator polynomial. The remainder of this division is attached to the data as a "checksum."

- The Application: At the receiving end, the data is divided again. If the remainder isn't zero, the system knows a bit was flipped during transmission. Primitive polynomials are chosen because they have the best mathematical properties for catching "burst errors" (multiple bits failing at once).

2. Cryptography: Stream Ciphers

In cybersecurity, primitive polynomials are used to build Stream Ciphers.

- How it works: An LFSR configured with a primitive polynomial generates a massive, "noise-like" bitstream. This stream is XORed with your private message (plaintext) to create an encrypted message (ciphertext).

- The Application: Since the sequence is determined by the primitive polynomial and a "seed" (the secret key), only someone with the same polynomial and seed can decrypt the message. This was the basis for the A5/1 cipher used in GSM mobile phones to encrypt voice calls.

3. Data Scrambling (Physical Layer)

High-speed digital cables (like HDMI, USB 3.0, and PCIe) use primitive polynomials for "scrambling."

- The Problem: If a computer sends a long string of zeros (e.g., an empty file), the electrical signal stays flat. This makes it hard for the hardware to stay synchronized.

- The Application: The data is XORed with a sequence generated by a primitive polynomial before being sent. This "scrambles" the data so it always looks busy and random, ensuring the hardware clock stays locked. The receiver simply XORs it again to get the original data back.

4. Built-In Self-Test (BIST) in VLSI

When companies like Intel or ISRO design microchips, they need a way to test millions of transistors quickly.

- The Application: They include a small LFSR on the chip. Using a primitive polynomial, the LFSR generates a Pseudo-Random Test Pattern that hits every part of the circuit. Another LFSR (acting as a signature analyzer) collects the results. If the final "signature" matches the expected value, the chip is marked as healthy.

5. Deep Space Communication (Reed-Solomon Codes)

For missions beyond Earth's orbit (like Mangalyaan or Chandrayaan), signals are incredibly weak.

- The Application: Primitive polynomials are used to construct Reed-Solomon codes. These codes add redundant "parity" bits to the message. Because of the mathematical properties of the primitive polynomial, the ground station at ISTRAC can actually fix missing or corrupted data without asking the satellite to resend the signal.

6. Multiple Access (CDMA)

As we have explored with NavIC, primitive polynomials are used to create Orthogonal Codes.

- The Application: By using different primitive polynomials (or different phases of the same one), thousands of mobile phone users or multiple satellites can talk on the same frequency at the same time without their signals "bleeding" into one another.

3.8  Generation of Gold Code used in NavIC

1. Definition and Functional Necessity

A Gold Code is a type of binary sequence used in Code Division Multiple Access (CDMA) systems. While a single LFSR (m-sequence) has excellent autocorrelation, it has poor cross-correlation properties. In a satellite constellation like NavIC, where all satellites transmit on the same frequency, Gold Codes are used because they provide a large family of unique codes that do not interfere with one another.

2. The Mathematical Foundation: Preferred Pairs

The "magic" of a Gold Code lies in the selection of two specific m-sequences of the same length, known as a Preferred Pair.

- The Criteria: Two primitive polynomials are a preferred pair only if their cross-correlation is bounded by three specific values:

  - (-1)

  - (-t(n))

  - t(n) - 2\)

- The Result: This ensures that the "noise" one satellite creates for another is predictable and minimal.

3. Hardware Architecture (The Two-Register Model)

The generation of a Gold Code requires a parallel architecture consisting of:

- G1 LFSR: A 10-stage shift register using a specific primitive polynomial.

- G2 LFSR: A 10-stage shift register using a different, but "preferred," primitive polynomial.

- The Master Clock: Both registers are driven by the same 1.023 MHz clock to ensure perfect synchronization.

- The Modulo-2 Combiner (XOR): The final output is created by XORing the bits from G1 and G2.

4. Step-by-Step Generation Logic

To generate a unique code for each satellite in the NavIC constellation, the following steps are followed:

Step A: Initialization (The Seed)

Both G1 and G2 are loaded with an initial "Seed" (usually all 1s). This defines the starting point of the sequence.

Step B: Phase Selection (The Identity)

This is the most critical step. To give Satellite 1 a different code than Satellite 2, ISRO does not change the registers. Instead, they apply a Phase Shift to the G2 sequence.

- This is done by taking the XOR of two specific "taps" within the G2 register.

- Each unique combination of taps produces a unique Gold Code.

Step C: Summation

The output of G1 and the phase-shifted output of G2 are fed into a final XOR gate. The resulting bitstream is the PRN Code for that specific satellite.

5. Worked Example (3-Bit System)

To illustrate the logic, we use a simplified 3-stage system (Length (N=7)).

A. Setup

- G1 Polynomial: (1 + X + X3) (Taps at 1, 3)  Sequence: 1 1 1 0 1 0 0

- G2 Polynomial: \(1 + X2 + X3) (Taps at 2, 3) Sequence: 1 1 1 0 0 1 0

B. Generation of Gold Code #1 (Zero Shift)

We XOR the two sequences directly:

- G1: 1 1 1 0 1 0 0

- G2: 1 1 1 0 0 1 0

- Result (GC1): 0 0 0 0 1 1 0

C. Generation of Gold Code #2 (Shifted G2)

We shift G2 by one position to the right (0 1 1 1 0 0 1):

- G1: 1 1 1 0 1 0 0

- G2 (shifted): 0 1 1 1 0 0 1

- Result (GC2): 1 0 0 1 1 0 1

Observation: GC1 and GC2 look completely different, yet they are part of the same Gold family.

6. Gold Codes in NavIC (SPS)

For SPS code generation, the two polynomials G1 and G2 are as defined below: G1: X10 + X3 + 1 and G2: X10 +X9 + X8 + X6 +X3 + X2 + 1 Polynomial G1 and G2 are similar to the ones used by GPS C/A signal. The G1 and G2 generators are realized by using 10 bits Maximum Length Feedback Shift Registers (MLFSR). The initial state of G2 provides the chip delay. The G1 register is initialized with all bits as 1. G1 and G2 are XOR‘ed for the generation of the final 1023 chip long PRN sequence. The time period of the PRN sequence is 1 millisecond. Table shows the identified SPS codes for various SVs for the SPS service

| PRN ID | Initial Condition for G2 Register (L5-SPS) | First 10 Chips in Octal (L5-SPS) | Initial Condition for G2 Register (S-SPS) | First 10 Chips in Octal (S-SPS) |
| --- | --- | --- | --- | --- |
| 1 | 1110100111 | 1647 | 0011001111 | 1470 |
| 2 | 0001001110 | 1731 | 0101111101 | 1202 |
| 3 | 1000110100 | 0713 | 1000110001 | 0716 |
| 4 | 0101110010 | 1215 | 0010101011 | 1524 |
| 5 | 1110110000 | 0117 | 1010010001 | 0556 |
| 6 | 0001101011 | 1624 | 0100101100 | 1123 |
| 7 | 0000010100 | 1753 | 0010001110 | 1561 |
| 8 | 0100110000 | 1317 | 0100100110 | 1331 |
| 9 | 0010011000 | 1547 | 1100001110 | 0361 |
| 10 | 1101000100 | 0733 | 1010111110 | 0501 |
| 11 | 0001000100 | 1663 | 1110010001 | 0156 |
| 12 | 1101111100 | 0203 | 1101101001 | 0226 |
| 13 | 1011010010 | 0455 | 0101000101 | 1272 |
| 14 | 0111101010 | 1025 | 0100001101 | 1362 |

Table 3.8 :  Sequence generation in NavIC for SPS

3.9 Optimization of Periodic Autocorrelation for GNSS Application using AI/ML Technique

To enhance the ranging accuracy of NavIC signals, this project explored the computational optimization of binary sequences to minimize the Peak Sidelobe Level (PSL). Two primary algorithms were developed in Python:

- Simulated Annealing (SA): Utilized a stochastic hill-climbing approach with a cooling schedule. By accepting occasionally worse solutions in the early stages (Metropolis Criterion), the algorithm effectively bypassed local minima to find sequences with significantly lower PSL than standard Gold codes.

- Genetic Algorithm (GA): Implemented a population-based evolutionary strategy. Through successive generations of selection, crossover, and mutation, the algorithm "bred" sequences that exhibited superior autocorrelation properties.

Key Optimization Logic: The periodic autocorrelation was computed efficiently in the frequency domain using Fast Fourier Transforms (FFT). The optimization was initialized using a Legendre sequence derived from the nearest prime, ensuring a high-quality mathematical starting point before heuristic refinement began.

Algorithm 1: Hybrid Simulated Annealing (SA)

This algorithm mimics the physical process of heating a material and slowly cooling it to remove defects and reach a "minimum energy" state (lowest PSL).

- Initialize:

  - Set the target length (N).

  - Generate a baseline sequence (S) using a Legendre sequence (Legendre seed).

  - Calculate initial (PSL) using FFT: (PSL_{current} = text{get_psl}(S)).

  - Set initial temperature (T = 45.0) and best (PSL) found so far.

- Iterative Optimization (15,000 cycles):

  - Perturb: Randomly select 1 to 3 bits in the sequence and flip them (multiply by (-1)).

  - Evaluate: Calculate the (PSL_{new}) of the modified sequence.

  - Acceptance Check (Metropolis Criterion):

    - If (PSL_{new} < PSL_{current}), accept the change immediately.

    - If (PSL_{new} geq PSL_{current}), calculate a probability (P = exp(frac{PSL_{current} - PSL_{new}}{T})). Accept the change only if a random number is less than (P).

  - Update Best: If the new sequence is the best ever found, save it as (S_{best}).

- Cooling: Reduce the temperature: (T = T \times 0.9998).

- Terminate: After 15,000 iterations, output the optimized sequence and report the reduction in PSL.

Algorithm 2: Genetic Algorithm (GA)

This algorithm uses evolutionary principles to "breed" better sequences over multiple generations.

- Population Initialization:

  - Create a starting population of 10 sequences.

  - Each member is a cyclically shifted version of the "Unified Legendre Seed."

- Generational Loop (60 generations):

  - Fitness Evaluation: Rank all 10 sequences based on their PSL (lowest is best).

  - Selection: Retain the top 2 sequences ("Elite") automatically for the next generation.

  - Reproduction (Crossover):

    - While the new population is not full, pick the best sequence (Parent 1) and a random high-performer (Parent 2).

    - Choose a random "Crossover Point."

    - Create a child by taking the first half from Parent 1 and the second half from Parent 2.

| Length of sequence | Initial PSL | Genetic Algorithm (GA-LS) Optimized PSL | Hybrid Simulated Annealing (HSA) Optimized PSL |
| --- | --- | --- | --- |
| 100 | 20 | 12 | 12 |
| 200 | 44 | 28 | 20 |
| 1023 | 85 | 73 | 67 |
| 1500 | 92 | 84 | 88 |
| 1800 | 124 | 108 | 104 |
| 2046 | 122 | 110 | 106 |
| 5115 | 201 | 193 | 187 |
| 10230 | 326 | 294 | 290 |

Table 3.9: Results of Optimization of Periodic Autocorrelation of binary sequences

Figure 3.9: Graph plot of PSL

CHAPTER 4

LEARNING OUTCOMES

4.1 . NavIC System & CDMA Architecture

- Outcome: Acquired a comprehensive understanding of India's independent satellite navigation infrastructure.

- Competency: Mastered the principles of Code Division Multiple Access (CDMA), specifically how unique PRN "signatures" allow multiple satellites to share the same frequency spectrum without mutual interference.

4.2. Abstract Algebra: Groups, Rings, and Galois Fields

- Outcome: Bridged the gap between abstract mathematical theory and digital signal design.

- Competency: Gained proficiency in Finite Field Arithmetic (GF(2n)), understanding how binary addition (XOR) and multiplication (AND) form the logical basis for error-free sequence generation.

4.3. Linear Feedback Shift Registers (LFSR)

- Outcome: Mastered the design and operation of high-speed hardware sequence generators.

- Competency: Learned to configure Fibonacci and Galois architectures and apply Primitive Polynomials to ensure the generation of Maximum Length Sequences (m-sequences) with a period of (2n-1).

4.4. PRN Code Generation

- Outcome: Developed the ability to generate and validate industry-standard satellite ranging codes.

- Competency: Understood the necessity of PRN Codes for multi-satellite constellations and learned to implement "Preferred Pairs" to maintain low cross-correlation bounds between different satellite IDs.

4.5. Python Implementation & Digital Simulation

- Outcome: Enhanced computational skills for simulating complex communication hardware.

- Competency: Leveraged NumPy and SciPy for high-performance signal analysis, specifically using Fast Fourier Transforms (FFT) to perform rapid periodic autocorrelation—a technique used in modern Software Defined Radios (SDR).

4.6. Sequence Optimization (SA & GA Algorithms)

- Outcome: Advanced from "algebraic generation" to "computational optimization" of signals.

- Competency: Successfully implemented Simulated Annealing (SA) and Genetic Algorithms (GA) to minimize the Peak Sidelobe Level (PSL) of sequences. Learned how to reduce multipath interference by mathematically refining the signal's "thumbtack" autocorrelation properties.

4.7. Primitive Polynomials

- Outcome: Understood the mathematical "recipe" required to ensure sequence reliability and uniqueness.

- Competency: Learned to identify and apply Primitive Polynomials of degree-10 to generate maximal-length cycles. I mastered how these polynomials prevent "short-cycling" and ensure that the signal visits every possible state (\(2^n-1\)) before repeating, which is critical for unambiguous ranging.

4.8. Gold Code Generation in NavIC

- Outcome: Developed expertise in the specific PRN code families used in the NavIC Standard Positioning Service (SPS).

- Competency: Mastered the structural generation of Gold Codes by XORing "Preferred Pairs" of m-sequences. I gained a deep understanding of why NavIC uses two 10-bit LFSRs (G1 and G2) and how varying the Phase Shift of the G2 register allows ISTRAC to assign a unique, non-interfering "fingerprint" to each satellite in the constellation.

4.9. Properties of Sequences in Satellite Communication

- Outcome: Gained a deep appreciation for the statistical requirements of space-grade signals.

- Competency: Evaluated and verified the four essential properties of PRN sequences:

  - Balance Property: Ensuring an equal distribution of 1s and 0s to eliminate DC bias in satellite transmitters.

  - Run-Length Property: Confirming that the sequence mimics "white noise" to spread signal energy across the spectrum (Spread Spectrum).

  - Autocorrelation: Exploiting the "thumbtack" peak to calculate nanosecond-level Time of Flight (ToF).

  - Cross-Correlation: Maintaining low interference levels between multiple satellites operating on the same frequency.

4.10. Strategic Implementation at ISTRAC

- Outcome: Understood the operational link between ground-station tracking and satellite signal structure.

- Competency: Learned how the Octal representation of initial conditions (as seen in ISRO documentation) is used to program ground-station receivers to instantly synchronize with NavIC satellites as they rise over the horizon.

CHAPTER 5

CONCLUSION

My internship at the ISRO Telemetry, Tracking and Command Network (ISTRAC) provided a profound opportunity to engage with the technical core of the Navigation with Indian Constellation (NavIC). During this period, I focused on the physical layer of satellite communication, specifically the design and optimization of Pseudo-Random Noise (PRN) codes. I mastered the mathematical foundations of Galois Field Theory and the use of Primitive Polynomials to configure Linear Feedback Shift Registers (LFSR). This allowed me to understand how ISRO generates maximal-length sequences that ensure every satellite has a unique, non-interfering digital signature.

I deeply explored the structural generation of Gold Codes, which are the operational standard for NavIC’s Standard Positioning Service. By studying the interaction between G1 and G2 registers, I learned how to manipulate code phases to maintain the critical properties of Balance, Run-length, and Autocorrelation. These properties are essential for spread-spectrum resilience and the high-precision ranging required for accurate global positioning. My work involved verifying these sequences through Python-based simulations, utilizing Fast Fourier Transforms (FFT) to analyze signal integrity and correlation spikes.

The most advanced phase of my internship involved the research and application of heuristic optimization techniques. I implemented Simulated Annealing (SA) and Genetic Algorithms (GA) to mathematically refine binary sequences, successfully reducing the Peak Sidelobe Level (PSL). This effort is critical for minimizing ranging errors caused by signal reflections in urban environments. Ultimately, this experience at ISTRAC bridged the gap between academic theory and space-grade engineering, providing me with a specialized skill set in navigation signal design while highlighting the strategic importance of NavIC in India’s quest for technological self-reliance.

REFERENCES

- Indian Space Research Organisation (ISRO). (2017). Indian Regional Navigation Satellite System: Signal-in-Space Interface Control Document (ICD) for Standard Positioning Service (SPS) - Version 1.1. ISRO-ISTRAC, Bangalore.

- Indian Space Research Organisation (ISRO). (2023). NavIC Signal-In-Space Interface Control Document (ICD) for Standard Positioning Service (SPS) in L1 Frequency. ISRO-ISTRAC, Bangalore.

- Teunissen, P. J., & Montenbruck, O. (Eds.). (2017). Springer Handbook of Global Navigation Satellite Systems. Springer Nature.

- Gold, R. (1967). Optimal binary sequences for spread spectrum multiplexing. IEEE Transactions on Information Theory, 13(4), 619-621.

- Golomb, S. W. (1981). Shift Register Sequences. Aegean Park Press.

- Betz, J. W. (2016). Engineering Satellite-Based Navigation and Timing: Global Navigation Satellite Systems Signals and Receivers. Wiley-IEEE Press.

- Jetzek, U. (2018). Galois Fields, Linear Feedback Shift Registers and their Applications. Carl Hanser Verlag GmbH Co KG.

- Sahoo, P. K., et al. (2016). Generation and implementation of IRNSS Standard Positioning Signal. Engineering Science and Technology, an International Journal, 19(3), 1381-1389.

- Grewal, A. S., et al. (2019). PRN Codes for Future NavIC L1 civil Signal. ICG-14 Working Group B, United Nations Office for Outer Space Affairs (UNOOSA).